Pill Dispenser V3

Complete Design Plan, Engineering Review & Build Workflow

Bipin — UPES Dehradun | Biomedical Engineering | 2026

Generated: March 2026 | KiCad 9.0.7 | ESP32-S3-WROOM-1

# 1. Project Overview

The Automatic Pill Dispenser is a portable, battery-powered medical device that dispenses medication on a programmed schedule. It uses a servo motor to rotate pill compartments, alerting the patient via buzzer and LED indicators when it is time to take medication.

## 1.1 What the device does

- Stores pills in a rotating compartment wheel (servo-driven)

- Keeps track of medication schedule via ESP32 RTC or BLE phone sync

- At scheduled time: rotates servo to correct compartment, buzzes, lights LED

- User presses button to confirm pill taken — LED turns off

- Runs on rechargeable Li-Ion battery, charges via USB-C

- OLED display shows current time, next dose time, and battery level

## 1.2 Miniaturization goals — V1 to V3

| Feature | V1 (Tinkercad) | V2 (KiCad) | V3 (Target) |
| --- | --- | --- | --- |
| Microcontroller | Arduino Uno ATmega328P | ESP32-S3 (wrong footprint) | ESP32-S3-WROOM-1 (correct) |
| Form factor | Breadboard ~20×10cm | PCB ~70×55mm (no outline) | PCB 60×50mm (clean) |
| Power | USB-A wall only | USB-C + Li-Ion | USB-C + Li-Ion (fixed) |
| Display | 16×2 LCD, 8 wires | I2C OLED, 2 wires | I2C OLED, 2 wires |
| Buzzer | THT 12mm | THT 12mm (V2 mistake) | SMD 5020 (5×5mm) |
| Servo power | 5V correct | 3.3V (WRONG - bug) | VBUS 5V (fixed) |
| LED resistors | 470Ω | Inconsistent: 100Ω–1kΩ | All 330Ω (standardized) |
| Wireless | None | BLE + WiFi (ESP32-S3) | BLE + WiFi (ESP32-S3) |
| Fab ready? | No (prototype) | No (multiple blockers) | Yes (goal) |

# 2. V2 Engineering Review — What We're Fixing

## 2.1 Critical bugs (board won't function without fixing these)

Bug 1: Servo connected to 3.3V rail — WRONG

The servo motor in V2 is powered from the 3.3V LDO output. This is incorrect and will cause the board to malfunction in the field.

- Servo motors draw 500–900mA at stall and 150–300mA during movement

- The AP2112K LDO is rated for 600mA total — shared with the ESP32 (~200mA WiFi peak)

- When the servo moves, it will brown out the 3.3V rail, crashing the ESP32

- FIX: Connect servo VCC directly to VBUS (5V from USB-C). Add 100µF decoupling cap on the 5V servo line. Only the PWM signal wire goes to ESP32 GPIO.

Bug 2: ESP32 symbol/footprint mismatch

The schematic uses ESP32-S3-MINI-1 symbol but the PCB has ESP32-S3-WROOM-1 footprint. These are physically different modules.

- MINI-1: 27mm × 34.3mm, 61 pads

- WROOM-1: 18mm × 20mm, 38 pads — smaller and more common

- FIX: Change schematic symbol to RF_Module:ESP32-S3-WROOM-1 to match PCB footprint

Bug 3: LED current limiting resistors are inconsistent

V2 has three different resistor values for LEDs: R1=100Ω, R2=470Ω, R13/R14=1kΩ. This is a design error.

- At 3.3V with 100Ω: (3.3 - 2.0) / 100 = 13mA — borderline; blue/white LEDs will be over-driven

- At 3.3V with 1kΩ: (3.3 - 2.0) / 1000 = 1.3mA — too dim, barely visible

- FIX: Standardize ALL LED current limiting resistors to 330Ω. This gives ~4mA which is clean and bright for status indication.

## 2.2 Important improvements (reliability and size)

Issue 4: AMS1117-3.3 is redundant

- AP2112K-3.3 already provides 3.3V at 600mA — sufficient for ESP32 + peripherals

- Having two LDOs on the same output rail risks reverse current and unstable regulation

- FIX: Remove AMS1117 from V3. If more current is needed later, upgrade AP2112K to AP2112K (1A variant)

Issue 5: 470µF THT electrolytic capacitor (C10)

- A large radial electrolytic sitting on an SMD board defeats the miniaturization goal

- It adds significant board height and is a mechanical weak point

- FIX: Replace with 100µF SMD electrolytic in 6.3×5.4mm package, or 2× 47µF 0805 ceramic

Issue 6: 12mm THT buzzer (BZ1)

- The original Buzzer_12x9.5RM7.6 footprint is massive for an SMD board

- FIX: Replace with MLT-5020 or similar SMD magnetic buzzer, 5×5mm footprint, same driving circuit (MMBT2222A stays)

Issue 7: Insufficient decoupling on 3.3V near ESP32

- ESP32-S3 WiFi/BLE radio causes brief ~300mA transient current spikes on 3.3V

- Only 4× 100nF bypass caps present — no bulk capacitance

- FIX: Add 1× 10µF ceramic 0805 cap on 3.3V rail, placed within 5mm of ESP32 module VDD pins

## 2.3 What's already correct in V2 — keep these

| Component | Status | Notes |
| --- | --- | --- |
| TP4056 charger | CORRECT | Standard LiPo/Li-Ion charger IC, proven design |
| AP2112K-3.3 LDO | CORRECT | 600mA, SOT-23-5, low dropout — good choice |
| 1N5819 Schottky diode | CORRECT | Reverse polarity protection + power OR-ing |
| CC1/CC2 resistors 5.1kΩ | CORRECT | Required for USB-C 5V/500mA negotiation per spec |
| I2C pull-ups R6/R7 4.7kΩ | CORRECT | Correct for 3.3V I2C at standard 100/400kHz |
| MMBT2222A buzzer driver | CORRECT | GPIO can't sink buzzer current directly — transistor needed |
| BOOT + EN pull-up 10kΩ | CORRECT | Matches Espressif reference design exactly |
| 100nF bypass caps ×4 | CORRECT | Keep, but add 10µF bulk as mentioned above |
| Antenna keep-out zone | CORRECT | No copper under ESP32 antenna — good RF practice |
| JST-PH battery connector | CORRECT | Standard, locking, correct 2mm pitch |
| R9/R10 100kΩ voltage divider | CORRECT | Battery level monitoring via ADC — document net labels |

# 3. V3 Complete Bill of Materials

## 3.1 Power section

| Ref | Value | Package | Purpose | Change from V2 |
| --- | --- | --- | --- | --- |
| J1 | USB-C HRO TYPE-C-31-M-12 | USB-C SMD | Power + programming port | No change |
| U3 | TP4056 | SOIC-8 | Li-Ion charger IC | No change |
| U4 | AP2112K-3.3 | SOT-23-5 | 3.3V LDO regulator | No change |
| U2 | REMOVE | — | AMS1117 removed | REMOVED — redundant |
| D3 | 1N5819 Schottky | D_SMA | Reverse polarity + OR-ing | No change |
| J4 | JST-PH 2-pin | JST-PH | Li-Ion battery connector | No change |
| SW4 | SPDT power switch | EG1224 | Main power switch | No change |
| C10 | 100µF 10V | 6.3×5.4mm SMD | Bulk power cap | CHANGED: was 470µF THT |
| C1 | 10µF 0805 | C_0805 | LDO input cap | No change |
| C2 | 10µF 0805 | C_0805 | LDO output cap | No change |
| C3 | 10µF 0805 | C_0805 | LDO output bulk | NEW: added for transients |
| R5 | 2kΩ 0603 | R_0603 | TP4056 charge current set (~580mA) | No change |
| R11 | 5.1kΩ 0603 | R_0603 | USB-C CC1 pull-down | No change |
| R12 | 5.1kΩ 0603 | R_0603 | USB-C CC2 pull-down | No change |
| R15 | 22Ω 0603 | R_0603 | USB D+ series resistor | No change |
| R16 | 22Ω 0603 | R_0603 | USB D− series resistor | No change |

## 3.2 Microcontroller section

| Ref | Value | Package | Purpose | Change from V2 |
| --- | --- | --- | --- | --- |
| U1 | ESP32-S3-WROOM-1 | RF_Module:ESP32-S3-WROOM-1 | Main MCU + WiFi/BLE | FIXED: symbol now matches footprint |
| C5 | 100nF 0603 | C_0603 | VDD bypass cap | No change |
| C6 | 100nF 0603 | C_0603 | VDD bypass cap | No change |
| C7 | 1µF 0603 | C_0603 | VDD bypass cap | No change |
| C8 | 100nF 0603 | C_0603 | VDD bypass cap | No change |
| C9 | 10µF 0805 | C_0805 | 3.3V bulk cap near ESP32 | NEW: added for RF transients |
| R8 | 10kΩ 0603 | R_0603 | BOOT (GPIO0) pull-up | No change |
| R17 | 10kΩ 0603 | R_0603 | EN (RESET) pull-up | No change |
| SW2 | Push 6mm | SW_PUSH_6mm | BOOT button | No change |
| SW3 | Push 6mm | SW_PUSH_6mm | RESET button | No change |

## 3.3 User interface and peripherals

| Ref | Value | Package | Purpose | Change from V2 |
| --- | --- | --- | --- | --- |
| J3 | 4-pin header 2.54mm | PinHeader_1x04 | I2C OLED connector | No change |
| J2 | 3-pin header 2.54mm | PinHeader_1x03 | Servo connector | CHANGED: VCC now from VBUS 5V |
| BZ1 | SMD Buzzer 5020 | Buzzer_5020_SMD | Audio alert | CHANGED: was 12mm THT |
| Q1 | MMBT2222A NPN | SOT-23 | Buzzer transistor driver | No change |
| D1 | Red LED 0805 | LED_0805 | STATUS indicator | No change |
| D4 | Red LED 0805 | LED_0805 | TP4056 charging indicator | No change |
| D5 | Green LED 0805 | LED_0805 | TP4056 standby indicator | No change |
| R1 | 330Ω 0603 | R_0603 | LED current limit | FIXED: was 100Ω |
| R2 | 330Ω 0603 | R_0603 | LED current limit | FIXED: was 470Ω |
| R13 | 330Ω 0603 | R_0603 | LED current limit | FIXED: was 1kΩ |
| R14 | 330Ω 0603 | R_0603 | LED current limit | No change (was already 1kΩ — standardized) |
| R4 | 1kΩ 0603 | R_0603 | Q1 base resistor | No change |
| SW1 | Push 6mm | SW_PUSH_6mm | USER button | No change |
| R6 | 4.7kΩ 0603 | R_0603 | I2C SDA pull-up | No change |
| R7 | 4.7kΩ 0603 | R_0603 | I2C SCL pull-up | No change |
| R9 | 100kΩ 0603 | R_0603 | Battery voltage divider (top) | No change — label net as VBAT_DIV |
| R10 | 100kΩ 0603 | R_0603 | Battery voltage divider (bottom) | No change — connect to ADC GPIO |

# 4. PCB Layout Rules for V3

## 4.1 Board specification

| Parameter | Value | Reason |
| --- | --- | --- |
| Board size | 60 × 50 mm | Fits all components with routing space |
| Layers | 2 (F.Cu + B.Cu) | Standard 2-layer, cost-effective at JLCPCB |
| PCB thickness | 1.6 mm | Standard, mechanical strength |
| Surface finish | HASL lead-free or ENIG | ENIG better for fine-pitch ESP32 pads |
| Min trace width | 0.2 mm signal, 0.5 mm power | JLCPCB supports 0.1mm min, 0.2mm is safe |
| Min drill | 0.3 mm | JLCPCB minimum — fix USB-C shield pads from V2 |
| Via size | 0.8 mm drill, 1.2 mm annular | Standard, easy to manufacture |
| Copper pour | GND pour on both layers | Reduces EMI, simplifies routing |
| Solder mask | Green (or black for look) | Standard default |

## 4.2 Component placement zones

Divide the 60×50mm board into 4 functional zones before placing a single component:

- Top-left zone (20×25mm): Power block — USB-C, TP4056, AP2112K, battery connector, diode, bulk caps

- Top-right zone (40×25mm): ESP32-S3-WROOM-1 module — keep-out zone for antenna extends 3mm beyond module edge on top

- Bottom-left zone (30×25mm): User interface — 3 buttons, 3 LEDs, buzzer, OLED header

- Bottom-right zone (30×25mm): Peripheral connectors — servo header, power switch, decoupling passives

## 4.3 Critical routing rules

USB D+/D− differential pair

- Route GPIO19 (D−) and GPIO20 (D+) as a differential pair

- Keep traces equal length — maximum 5mm length mismatch

- Minimum 0.2mm spacing between the two traces

- No vias on D+/D− — route entirely on front copper

- Keep away from high-current power traces

ESP32 antenna keep-out

- The WROOM-1 module has a PCB antenna at the top edge

- No copper (traces, pours, fills) on either layer under the antenna region

- Keep-out extends 3mm beyond the module edge where the antenna protrudes

- This is already correctly set in V2 — preserve in V3

Decoupling capacitors

- Place each 100nF bypass cap within 2mm of its corresponding VDD pin on the ESP32 footprint

- Route cap GND directly to ground pour with a short via — not through a long trace

- 10µF bulk cap can be up to 5mm away from the module

Power traces

- 3.3V rail: minimum 0.5mm trace width

- VBUS 5V rail: minimum 0.8mm trace width (servo can pull 500mA)

- Battery input: minimum 0.5mm trace width

- GND: use copper pour — do not route individual GND traces

Servo connector

- Place J2 (servo) on a board edge for easy cable routing to the mechanical assembly

- The 5V trace from VBUS to servo VCC pin must be at least 0.8mm wide

- PWM signal from GPIO9 is a standard 0.2mm signal trace

# 5. Complete Build Workflow

## Step 1 — Create new KiCad V3 project

- Open KiCad 9.0.7

- File → New Project → name it Pill_Dispenser_V3

- Save to: D:\UPES\Projects\Pill_Dispenser_V3\

- This creates a clean .kicad_sch and .kicad_pcb — no broken extends, no legacy issues

## Step 2 — Draw schematic

- Open Schematic Editor

- Add power symbols: +3.3V, +5V (VBUS), GND, VBAT

- Add U1: RF_Module:ESP32-S3-WROOM-1 (not MINI-1)

- Add U3: Battery_Management:TP4056-42-ESOP8

- Add U4: Regulator_Linear:AP2112K-3.3 (SOT-23-5)

- Wire all nets as described in Section 3 BOM

- Add PWR_FLAG symbols on +3.3V and +5V to suppress ERC warnings

- Run ERC (Inspect → Electrical Rules Checker) — fix until 0 errors

## Step 3 — Assign all footprints

- Tools → Assign Footprints

- U1: RF_Module:ESP32-S3-WROOM-1

- U3: Package_SO:SOIC-8_3.9x4.9mm_P1.27mm

- U4: Package_TO_SOT_SMD:SOT-23-5

- BZ1: Buzzer_Beeper:Buzzer_5020_SMD (not Buzzer_12x9.5RM7.6)

- All R: Resistor_SMD:R_0603_1608Metric

- All C power: Resistor_SMD:C_0805_2012Metric

- All C bypass: Capacitor_SMD:C_0603_1608Metric

- All LEDs: LED_SMD:LED_0805_2012Metric

- Q1: Package_TO_SOT_SMD:SOT-23

## Step 4 — Update PCB from schematic

- Open PCB Editor

- Tools → Update PCB from Schematic (F8)

- All footprints load as unplaced pile — do NOT place yet

## Step 5 — Draw Edge.Cuts FIRST

- Select Edge.Cuts layer

- Draw a rectangle: (0,0) to (60,50) mm

- This is your board outline — draw it BEFORE placing any components

- Verify in DRC that the outline is a closed polygon — no gaps

## Step 6 — Place components

- Start with ESP32-S3-WROOM-1 — top-right, antenna facing away from board center

- Draw keep-out zone on F.Cu and B.Cu under antenna area

- Place power block (USB-C, TP4056, AP2112K) — top-left corner

- Place decoupling caps immediately adjacent to ESP32 VDD pins

- Place UI components (buttons, LEDs, buzzer, OLED header) — bottom edge for easy access

- Place connectors (servo, battery) on board edges

## Step 7 — Route traces

- Route GND copper pour last — add at the end with Zone Fill

- Route power traces first (3.3V, 5V) with correct widths

- Route USB D+/D− as matched-length differential pair

- Route signal traces (I2C, PWM, GPIO)

- Fill GND zones on F.Cu and B.Cu — Inspect → Board Statistics to confirm coverage

## Step 8 — DRC and verification

- Inspect → Design Rules Checker

- Target: 0 unconnected, 0 errors

- Acceptable: lib_footprint_mismatch warnings (non-blocking) — add to exclusions

- Verify: Edge.Cuts closed, no copper under ESP32 antenna, USB-C drill ≥ 0.3mm

- Run ERC one more time on schematic — confirm still 0 errors

## Step 9 — Export Gerbers for JLCPCB

- File → Plot — select Gerber format

- Output layers: F.Cu, B.Cu, F.Mask, B.Mask, F.Silkscreen, B.Silkscreen, Edge.Cuts

- File → Fabrication Outputs → Drill Files — Excellon format, PTH + NPTH

- Zip all Gerber + drill files together

- Upload ZIP to jlcpcb.com → 2-layer board → standard specs → order 5 pieces (~$2)

# 6. KiCad MCP Commands Reference

Your MCP server is running with 99 tools and KiCad Python validated. Use these commands in VS Code with Claude to automate V3 steps.

## 6.1 Project setup

| Task | MCP command to give Claude |
| --- | --- |
| Create new project | Use KiCad MCP to create a new project named Pill_Dispenser_V3 at D:\UPES\Projects\Pill_Dispenser_V3 |
| Draw board outline | Use KiCad MCP to draw Edge.Cuts rectangle from (0,0) to (60,50) on Pill_Dispenser_V3.kicad_pcb |
| Add keep-out zone | Use KiCad MCP to add a keep-out zone on F.Cu and B.Cu under the ESP32 antenna area at coordinates (38,0) to (60,15) |

## 6.2 Schematic commands

| Task | MCP command to give Claude |
| --- | --- |
| Check all nets | Use KiCad MCP to list all nets in Pill_Dispenser_V3.kicad_sch and check for unconnected pins |
| Verify footprints | Use KiCad MCP to list all components and their assigned footprints in Pill_Dispenser_V3.kicad_sch |
| Check for mismatch | Use KiCad MCP to compare schematic symbols to PCB footprints and report any mismatches |

## 6.3 PCB commands

| Task | MCP command to give Claude |
| --- | --- |
| Run DRC | Use KiCad MCP to run DRC on Pill_Dispenser_V3.kicad_pcb and return all errors and unconnected items as JSON |
| Check USB-C drills | Use KiCad MCP to find all pads on J1 and report their drill hole sizes — flag any below 0.3mm |
| Move ESP32 to position | Use KiCad MCP to move footprint U1 to position (40,12) with rotation 0 on Pill_Dispenser_V3.kicad_pcb |
| Get board stats | Use KiCad MCP to report board dimensions, component count, and trace statistics for Pill_Dispenser_V3.kicad_pcb |

# 7. Firmware Notes

## 7.1 Programming via USB-C (same port)

The ESP32-S3 has native USB. GPIO19 = D−, GPIO20 = D+ connect directly to the USB-C connector. No CH340, CP2102, or any USB-to-UART chip needed.

## 7.2 Flash procedure

To enter bootloader mode and flash firmware:

- Hold BOOT button (SW2 — GPIO0 to GND)

- Press and release RESET button (SW3 — EN to GND)

- Release BOOT button — ESP32 is now in download mode

- Select correct COM port in Arduino IDE or run esptool

- After flash completes, press RESET once to run new firmware

## 7.3 Arduino IDE setup for ESP32-S3

- Install ESP32 board package: https://raw.githubusercontent.com/espressif/arduino-esp32/gh-pages/package_esp32_index.json

- Board: ESP32S3 Dev Module

- USB Mode: USB-OTG (TinyUSB)

- Upload Mode: UART0 / Hardware CDC

- Flash Size: 8MB (WROOM-1 default)

## 7.4 Key GPIO assignments for firmware

| GPIO | Function | Notes |
| --- | --- | --- |
| GPIO0 | BOOT button | Active LOW — 10kΩ pull-up to 3.3V |
| GPIO4 | I2C SDA (OLED) | 4.7kΩ pull-up on PCB |
| GPIO5 | I2C SCL (OLED) | 4.7kΩ pull-up on PCB |
| GPIO9 | Servo PWM | 50Hz, 1–2ms pulse width |
| GPIO10 | Buzzer (via Q1) | Drive HIGH to activate buzzer |
| GPIO11 | STATUS LED | Drive HIGH = LED ON (330Ω in series) |
| GPIO12 | USER button | Active LOW — add pull-up in firmware |
| GPIO19 | USB D− | Do NOT use as GPIO — reserved for USB |
| GPIO20 | USB D+ | Do NOT use as GPIO — reserved for USB |
| GPIO34/35 | Battery ADC | 100kΩ voltage divider — read battery level |
| EN | RESET | Active LOW — 10kΩ pull-up to 3.3V |

# 8. Pre-Fab Checklist — V3

Run through every item before uploading Gerbers to JLCPCB:

## Schematic

| # | Check | Status |
| --- | --- | --- |
| 1 | ESP32 symbol is RF_Module:ESP32-S3-WROOM-1 (not MINI-1) | [ ] |
| 2 | Servo VCC net connects to VBUS 5V, not 3.3V | [ ] |
| 3 | All LED resistors are 330Ω | [ ] |
| 4 | AMS1117 removed from schematic | [ ] |
| 5 | CC1 and CC2 both have 5.1kΩ to GND | [ ] |
| 6 | D+/D− connect to GPIO19/GPIO20 | [ ] |
| 7 | PWR_FLAG on +3V3 and +5V nets | [ ] |
| 8 | ERC shows 0 errors | [ ] |

## PCB

| # | Check | Status |
| --- | --- | --- |
| 1 | Edge.Cuts is a closed 60×50mm rectangle | [ ] |
| 2 | No copper under ESP32 antenna keep-out zone | [ ] |
| 3 | USB-C shield pad drills ≥ 0.3mm | [ ] |
| 4 | All decoupling caps within 2mm of ESP32 VDD pins | [ ] |
| 5 | D+/D− traces equal length, no vias | [ ] |
| 6 | 5V servo trace width ≥ 0.8mm | [ ] |
| 7 | 3.3V power trace width ≥ 0.5mm | [ ] |
| 8 | GND copper pour filled on both layers | [ ] |
| 9 | Silkscreen labels visible and not overlapping pads | [ ] |
| 10 | DRC shows 0 errors, 0 unconnected | [ ] |

## Gerber export

| # | Check | Status |
| --- | --- | --- |
| 1 | Gerbers exported: F.Cu, B.Cu, F.Mask, B.Mask, F.Silk, B.Silk, Edge.Cuts | [ ] |
| 2 | Drill file exported: Excellon format, PTH + NPTH separate | [ ] |
| 3 | All files zipped into single ZIP | [ ] |
| 4 | ZIP previewed in JLCPCB Gerber Viewer — board outline visible | [ ] |
| 5 | Layer stack confirmed: 2-layer, 1.6mm, HASL or ENIG | [ ] |

Once all checkboxes are ticked — upload the ZIP to jlcpcb.com. 5 boards for approximately $2 + shipping. Standard turn: 3–5 business days.